INFORMATION TECHNOLOGY INSTITUTE

Analog IC Design Track

Analog Integrated Circuit Design

Lab 11

Mini Project 02

Fully-Differential Folded Cascode OTA

Table of Contents

1	Part 1: gm/ID Design Charts	4

1.1	NMOS	4

1.2	PMOS	6

2	PART 2: OTA Design	8

2.1	Specs:	8

2.2	Sizing:	9

2.2.1	Input Pair:	9

2.2.2	NMOS Current source:	9

2.2.3	Cascode NMOS:	10

2.2.4	Cascode PMOS:	10

2.2.5	Tail PMOS:	10

2.2.6	PMOS Current Source:	10

3	PART 3: Open-Loop OTA (Behavioral CMFB)	12

3.1	OP Analysis:	12

3.2	Diff small signal ccs:	13

4	PART 4: Open-Loop OTA Simulation (Actual CMFB)	16

4.1	CMFB Design:	16

4.2	OP Analysis:	17

4.3	Diff small signal ccs:	19

5	PART 5: Closed Loop Simulation (AC and STB )	20

5.1	OP Analysis:	21

5.2	Differential closed-loop response:	22

5.3	Differential and CMFB loops stability (STB analysis):	23

6	PART 6: Closed Loop Simulation (Transient):	27

6.1	Differential and CMFB loops stability:	27

6.2	Differential and CMFB loops stability (transient analysis):	29

6.3	Output swing:	29

6.3.1	CMIR Range:	31

7	Specs Achieved	31

8	Bonus Part:	32

8.1	Bias Circuit:	32

8.1.1	AC analysis :	34

9	FOM	34

9.1	Total Current	34

9.1.1	OTA	34

9.1.2	CMFB	35

9.2	AREA	35

9.2.1	OTA	35

9.2.2	CMFB	35

Part 1: gm/ID Design Charts

NMOS

Figure 1: NMOS (VA) vs. gm/ID.

Figure 2: NMOS VGS vs. gm/ID.

Figure 3: NMOS Transit Frequency (fT) vs. gm/ID

Figure 4: NMOS W/ID vs. gm/ID.

PMOS

Figure 5: PMOS (VA) vs. gm/ID.

Figure 6: PMOS VGS vs. gm/ID.

Figure 7: PMOS Transit Frequency (fT) vs. gm/ID

Figure 8: PMOS W/ID vs. gm/ID.

PART 2: OTA Design

Specs:

| Parameter | Specification (GF180MCU) |
| --- | --- |
| Supply Voltage (VDD) | 2.5 V |
| Closed Loop Gain (ACL) | 2 V/V |
| Phase Margin (at target ACL) | ≥70∘ |
| CM Input Range - Low | ≤0 V |
| CM Input Range - High | ≥1 V |
| Differential Output Swing | 1.2 V peak-to-peak |
| Load Capacitance (CL) | 500 fF |
| DC Loop Gain | 60 dB |
| Closed-Loop Settling Time (1% error) | 100 ns |

First, we will choose pmos input pair since the CM input low is 0V.

Taking the loading effect into consideration:

β=CFCF+CIN=1p1p+2p=13

LG=60dB→1000=AOL*β→AOL=3000

AOL=CINCF+CIN*Ao=2p1p+2p*Ao=23*Ao

BWOL= BWOTA

Gain-Bandwidth Product (GBW) Relationships

CinCin+CfAo⋅BWOTA=Acl⋅BWcl

GBWOTA=3⋅BWcl

GBWOTA=gm2πCout

Cout=CL+Cf∥Cin=1.167 pF

Using the 1% error settling time:

e-Tτ*100=1→τ=21.7ns

BWCL=12πτ=7.33MHz

GBWCL=ACL*BWCL=BWCLBeta= 22MHz=gmin2πCout,tot→gmin≈161μS

We will take gmin≈165μS

Output Resistance is Rout=ro3gm3ro2∥ro3∥ro4gm4ro5

Sizing:

Input Pair:

Let L = 300nm we will take min L to reduce Cgg.

gmin≈165μS

Assume gm/ID=16.5

So we will get that the current is  ID=10μA

W=20.88 μm

Figure 9: input pair

NMOS Current source:

Figure 10: NMOS CS

choosing large L for bigger ro to reduce noise and mismatch.

L=1μm

And biasing in SI for lower gm and lower noise

gmID=8

we get

W=5.52 μm

Cascode NMOS:

Figure 11: Cascode Nmos

Choosing Large L= 1 μm and

gmID=9

We get W = 3.56 u

Cascode PMOS:

Choosing Large L= 1 μm and

gmID=9

Figure 12: Cascode Pmos

We get W = 15.3 u

Tail PMOS:

Figure 13: Tail PMOS

Choosing Large L= 1 μm  for lower mismatch and

gmID=7

To lower gm for lower noise

We get W = 17.68u

Figure 14: PMOS CS

PMOS Current Source:

Same as the tail

We get W = 8.84 u

Bias Voltages:

VCASCN=VGScascN+VMN*=1.11V

VCASCP=VDD-VGScascP-VMP*=1.24V

Since we didn’t take the body effect into consideration so we must Choose slighter higher and lower values to ensure transistor operate in saturation:

VCASCN=1.2V

VCASCP=1..15V

Table 1: Final Sizing

| Transistor | Function | Type | ID​ (μA) | gm​/ID​ | L (μm) | W (μm) |
| --- | --- | --- | --- | --- | --- | --- |
| M1 | Input Pair | PMOS | 10 | 16.5 | 0.3 | 20.88 |
| M2 | Sink Mirror | NMOS | 20 | 8.0 | 1.0 | 5.52 |
| M3 | Cascode | NMOS | 10 | 9.0 | 1.0 | 3.56 |
| M4 | Cascode | PMOS | 10 | 9.0 | 1.0 | 15.32 |
| M5 | Current Source | PMOS | 10 | 7.0 | 1.0 | 8.84 |
| M6 | Tail Source | PMOS | 20 | 7.0 | 1.0 | 17.68 |

PART 3: Open-Loop OTA (Behavioral CMFB)

Figure 12: Open Loop Schematic

OP Analysis:

Figure 13: OP parameters before

| Transistor(s) | Type | Function | ID​ (µA) | VGS​ | VDS​ | VDSAT​ | VTH​ | gm​ | gds​ | Region |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M0, M1 | PMOS | Input Pair | -10.42 | -655.31 | -981.58 | -93.59 | -620.78 | 170.27 | 2.29 | SAT |
| M2 | PMOS | Tail Source | -20.84 | -1090.47 | -1343.89 | -224.40 | -791.94 | 144.23 | 0.35 | SAT |
| M3, M4 | PMOS | Top Sources | -9.70 | -1035.41 | -303.95 | -220.77 | -753.87 | 67.40 | 1.06 | SAT |
| M5, M6 | PMOS | Cascodes | -9.70 | -1046.05 | -851.12 | -161.47 | -913.14 | 109.96 | 0.55 | SAT |
| M9, M10 | NMOS | Bottom Sinks | 20.11 | 893.86 | 274.73 | 216.08 | 687.69 | 157.18 | 4.28 | SAT |
| M12, M13 | NMOS | Cascodes | 9.70 | 925.27 | 970.28 | 169.18 | 728.73 | 100.92 | 4.72 | SAT |
| M8 | PMOS | Bias Diode | -10.00 | -1035.41 | -1035.41 | -220.78 | -753.87 | 70.41 | 0.21 | SAT |
| M7 | PMOS | Bias Mirror | -10.10 | -1035.41 | -1006.14 | -220.78 | -753.87 | 71.10 | 0.15 | SAT |
| M11 | NMOS | Bias Diode | 10.10 | 893.86 | 893.86 | 215.17 | 685.09 | 79.65 | 0.29 | SAT |

What is the CM level at the OTA output?

(VOCM) is 1.245V. which is the reference voltage (VREF) to approximately half of the 2.5V supply.

What are the differential input and output voltages of the error amplifier? What is the relation
between them?

The  inputs of CMFB are common-mode voltage (VCM) and the target reference (VREF). Vdiff = 5mV. Its output is VCTRLP. Because the ideal error amplifier is assigned a gain of 1, the relationship is defined by:

VCTRLP=VBP+1⋅VCM-VREF

The simulation confirms this: with VOCM=1.245V, the generated VCTRLP is at 1.46V, which is almost perfectly aligned with the bias voltage VBP of 1.465V.

Diff small signal ccs:

Figure 14: Ac parameters before resizing

As we see the open loop gain is very high more than required (3000), to reduce it we need to reduce Rout.

For the cascode devices we will reduce gm and ro so we will reduce the W and L

For Cascode NMOS:

Reduce W from 3.56u to 2.54u and L from 1u to 0.5u.

For Cascode PMOS:

Reduce W from 15.3u to 10u and L from 1u to 0.5u.

Figure 15: AC parameters after resizing

The PM is = 180 – 94 = 85

Also I checked the OP parameters again

Figure 16: OP after resizing

Figure 17: Differential open-loop magnitude response (dB) vs. frequency.

Figure 18: Differential open-loop phase response vs. frequency.

PM = 85°

Hand Analysis:

ROUT=gmcascnrocascnrmn//roinp//gmcascprocascpromp≈19MΩ

AOL=GmROUT=170μ*19M=3.23K

BWOL=12πROUTCOUT≈16.75KHz

GBWOL=UGF=AOL*BWOL=54MHz

|  | Simulation | Hand Analysis |
| --- | --- | --- |
| Aol | 72.18 dB (4063 V/V) | 3230 |
| BW | 12.46 kHz | 16.75KHz |
| GBW | 50.62 MHz | 454MHz |
| UGF | 50.77 MHz | 54MHz |

PART 4: Open-Loop OTA Simulation (Actual CMFB)

CMFB Design:

I designed the CMFB circuit to have 25% of the current budget of the OTA which is 10uA so every branch will have current = 2.5 uA.

First lets estimate the sensing resistance , in worst case the ∆Vout=1.2 if I choose the R = 50k as in the lab which makes Ix = 1.2/100k = 12u  we must have a  current in the common drain larger than 12 u but I designed to make it 2.5u so we will increase the R to be 250k instead of 50k that makes Ix = 2.4uA and that’s fine.

R ≥ 0.62.5u

R≥240k

I will take R = 250k

The Pmos current sources will have a width W = 2.21u and L = 1u which is half width of the pmos CS of main OTA.

For the tail  NMOS CS will have a width equals the quarter of the NMOS CS of the OTA W=1.38u

For the NMOS inputs of the differential pair we will sweep over W to make the gm of the diff pair equals the gm of the active loads to have a gain of 1.

We found the equal gm at W =0.6u.

Also since Vout had a positive shift because of the Common drain , so we will shift the Vref by the same shift so we will use same sizing.

Calculating new Output Swing:

minimum Output Swing=VCASCN*+VMN*≈0.5

maximum Output Swing=VDD-VGSP-VP*=1.5

Middle of Output Swing=VREFnew≈1V

OP Analysis:

Figure 19: DC operating point of the actual CMFB

Figure 20: DC operating point of the main folded-cascode OTA

Figure 21: VOCM where VREF2 is VREF after shifting

Table 2: Main OTA & Biasing DC Operating Point

| Transistor(s) | Function | ID​ (µA) | VGS​ (mV) | VDS​ (mV) | VDSAT​ (mV) | VTH​ (mV) | gm​ (µS) | gds​ (µS) | Region |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M0, M1 | Input Pair | 10.41 | 656.20 | 882.81 | 93.77 | 620.72 | 170.18 | 2.29 | SAT |
| M2 | Tail Source | 20.82 | 1040.37 | 1343.80 | 224.33 | 783.94 | 144.18 | 0.35 | SAT |
| M3, M4 | Top Sources | 9.70 | 1035.41 | 304.74 | 220.77 | 753.87 | 67.42 | 1.64 | SAT |
| M5, M6 | Cascodes | 9.70 | 1045.26 | 1180.98 | 160.85 | 913.18 | 107.32 | 0.48 | SAT |
| M9, M10 | Bottom Sinks | 20.11 | 893.86 | 273.39 | 216.06 | 687.68 | 157.06 | 4.40 | SAT |
| M12, M13 | Cascodes | 9.70 | 926.61 | 740.89 | 169.21 | 798.62 | 100.59 | 0.82 | SAT |
| M8 | Bias Diode | 10.00 | 1035.41 | 1035.41 | -220.78 | 753.87 | 70.41 | 0.21 | SAT |
| M7 | Bias Mirror | 10.10 | 1035.41 | 1006.14 | 220.78 | 753.87 | 71.10 | 0.15 | SAT |
| M11 | Bias Diode | 10.10 | 893.86 | 893.86 | 215.17 | 685.09 | 79.65 | 0.29 | SAT |

| Transistor(s) | Function | ID (µA) | VGS (mV) | VDS (mV) | VDSAT (mV) | VTH (mV) | gm (µS) | gds (µS) | Region |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CD Buffers | Level Shifters | 2.43 | 847.68 | 1846.96 | 106.17 | 786.27 | 36.43 | 0.11 | SAT |
| CD Bias | Buffer Sources | 2.43 | 1035.41 | 653.04 | 221.51 | 782.16 | 16.91 | 0.08 | SAT |
| Err Amp In | Diff Pair | 2.18 | 1125.04 | 758.87 | 233.92 | 903.10 | 16.65 | 0.08 | SAT |
| Err Amp Load | Diode Loads | 2.18 | 1019.21 | 1019.21 | 209.93 | 782.16 | 16.09 | 0.05 | SAT |
| Err Amp Tail | Tail Source | 4.72 | 893.86 | 721.92 | 213.37 | 682.56 | 37.03 | 0.18 | SAT |
| VREF Buffer | Level Shifter | 2.43 | 847.57 | 1867.57 | 106.11 | 786.25 | 36.42 | 0.11 | SAT |

Table 3: CMFB Circuit DC Operating Point

What is the CM level at the OTA output? Why?

VoCM = 1.005 which is almost equal to Vref=1, the CMFB senses the average and compares it to Vref and return to the tail CS forcing Vocm to follow Vref.

What are the differential input and output voltages of the error amplifier? What is the relation
between them?

VDIFFIN=1.868-1.853= 15mV

VDIFFOUT=1.465-1.46=5 mV

VDIFFOUTVDIFFIN=0.333

Diff small signal ccs:

Figure 22: Differential open-loop magnitude response (dB) with actual CMFB loading

Figure 23: Differential open-loop phase response with actual CMFB loading

Figure 24: AC Values

PM = 180-94=85°

| Parameter | Behavioral CMFB | Actual CMFB |
| --- | --- | --- |
| DC Gain (A0) | 72.18 dB | 71.16 dB (3.615k V/V) |
| -3dB Bandwidth (BW) | 12.46 kHz | 13.92 kHz |
| Unity Gain Freq. (UGF) | 50.77 MHz | 50.20 MHz |
| Gain-Bandwidth (GBW) | 50.62 MHz | 50.31 MHz |
| Phase Margin (PM) | 85.6° | 85.5° |

PART 5: Closed Loop Simulation (AC and STB )

Figure 25: Closed Loop Schematic

OP Analysis:

Figure 26: Folded OP

Figure 27: CMFB OP

Figure 28: OP

What is the CM level at the OTA output? Why?

It equals to 1.003 almost 1 as Vref.

What is the CM level at the OTA input? Why?

Also It equals to 1.003 almost 1 as Vref. Follows the output by the feedback.

Differential closed-loop response:

Figure 29: Closed-loop differential magnitude response (dB).

Figure 30: Closed-loop differential phase response

Figure 31:AC

GBWCL=GBWLoaded* 23

GBWloaded=23 MHz=gm2πCtot     from part 4

SO

GBWCL=15.33 MHz

So our GBW meets the specs

CinCin+CfAo⋅BWOTA=Acl⋅BWcl

BWcl=GBWloaded3=7.67 MHz

Differential and CMFB loops stability (STB analysis):

Figure 32: Differential Loop Gain magnitude (dB).

Figure 33: Differential Loop Gain phase

Figure 34: Common-Mode Loop Gain magnitude (dB)

Figure 35:Common-Mode Loop Gain phase

Figure 36: LG overlaid

Figure 37: LG phase overlaid

Figure 38: Differential Loop Gain

Figure 39: CM Loop Gain

| Parameter | Differential Loop | Common-Mode Loop |
| --- | --- | --- |
| DC Loop Gain (LG) | 61.65 dB (1.223k V/V) | 60.65 dB (1.079k V/V) |
| Loop Gain GBW | 7.168 MHz | 4.63 MHz |
| Phase Margin (PM) | 89.44° | 85.06° |

Compare GBW and PM of diff and CM loops. Comment.

|  | Diff | CM |
| --- | --- | --- |
| GBW | 7.168 | 4.63 |
| PM | 89 | 85 |

The differential loop has a higher Loop Gain GBW (7.168 MHz) compared to the CM loop (4.63 MHz). This is expected. The differential signal path is short and driven by the massive gm of the main input pair. The CM signal path travels through the CD  and the error amplifier.

Compare DC LG and GBW of the diff loop with those obtained from open-loop simulation. Comment.

|  | LG | OL |
| --- | --- | --- |
| GBW | 7.168 | 50 |
| DC LG | 61.65 dB (1.223k V/V) | 71dB |

In the open-loop simulation (Part 4), the DC gain was 71.16 dB and the GBW was 50.31 MHz. In this STB simulation, the DC Loop Gain drops to 61.65 dB. This exact ≈9.5 dB drop perfectly matches the feedback factor (β) introduced by the capacitive divider: β=Cf/Cs+Cf=1pF/3pF=1/3, and 20log101/3=-9.54 dB. The STB GBW (7.168 MHz) is the BWcl which is the GBWloaded= 23Beta=7.67MHz .

PART 6: Closed Loop Simulation (Transient):

Differential and CMFB loops stability:

Figure 40:  VINP, VINN, VOUTP, VOUTN, and VOCM overlaid

Figure 41:  VINP, VINN, VOUTP, VOUTN, and VOCM

Do you notice any differential/CM ringing? Are both loops stable with adequate PM?

No ringing just a little overshoot in the VOCM as we have high PM.

Figure 42: settling time before

To fix the settling time we should increase the GBW we can do that by increasing the width of the input pair to 30u instead of 21u and also increase the VCASCN to 1.3 instead of 1.2 and reduce VCASCP to 1 instead of 1.15 to increase the ro thus increasing Rout.

Figure 43: settling time after

Settling time = 98 ns which meets the specs.

Differential and CMFB loops stability (transient analysis):

Figure 44: Transient response to a 100mV common-mode step pulse

No ringing in CM or DIFF there is a little overshoot due to the capacitors where in transitions which is high frequency the caps become short circuit this only in transition which is high freq.

Output swing:

Figure 45: Transient output swing over three complete periods.

Figure 46: Zoomed-in view of the sinusoidal input and output waveforms.

Figure 47: VIDIFF and VODIFF overlaid

As we see when the Vin amplitude = 150m the Vout is 300m that’s because the closed loop gain = 2

Figure 48: Vout peak to peak

As we see   				 Vin_pp=600m

Vout_pp = 1.2V

So Av= Vout_pp/Vin_pp= 2

I did rough trans analysis by putting the amplitude of input = 1.1 to know what can my circuit do

Figure 49: MAX peak to peak

As we see the circuit clipped at 2V which is 4V peak to peak.

CMIR Range:

I swept over Vicm and plotted GBW to know the CMIR

Figure 50: CMIR

Vin_low = -350m

Vin_high = 1.9

Which meets the specs !

Specs Achieved

| Specification | Target Value | Achieved Result | Status |
| --- | --- | --- | --- |
| Supply Voltage (VDD) | 2.5 V | 2.5 V | Achieved |
| Closed-Loop Gain (ACL) | 2 V/V | 1.997 V/V | Achieved |
| Phase Margin (Differential) | ≥70∘ | 89.44° | Achieved |
| Phase Margin (Common-Mode) | ≥70∘ | 85.06° | Achieved |
| CM Input Range - Low | ≤0 V | -350 mV | Achieved |
| CM Input Range - High | ≥1 V | 1.9 V | Achieved |
| Differential Output Swing | ≥1.2 Vpk-pk | 1.656 Vpk-pk (4 max Vpk-pk) | Achieved |
| Load Capacitance (CL) | 500 fF | 500 fF | Achieved |
| DC Loop Gain | ≥60 dB | 61.65 dB | Achieved |
| 1% Settling Time | ≤100 ns | 98 ns (Rising Edge) | Achieved |

Bonus Part:

Bias Circuit:

We will use magic battery for VCASCN and VCASCP

In the left circuit (VCASCN) we will sweep over W or L to get Vgs= VCASCN =1.2 and the same for the right circuit we will sweep till we get VCASCP= 1.15

Also the diode connected devices is the same sizing the cascode devices we just sweep over the device we get VGS from.

I choose L and swept over W

NMOS: L= 10u

PMOS: L = 2u

Figure 51: VCASCP

Figure 52: VCASCN

So this is the final sizing of the magic battery.

i designed the 2 magic batteries so that the total current of them equals 5u, as we see I=4.97u.

Figure 53: Magic battey OP

Here is the final bias voltages we got.

AC analysis :

I run Ac analysis for closed loop using the bias voltages we got

Which is the same results when we used a biasing voltage dc.

FOM

Total Current

OTA

Tail Source (M2): 20.82 μA

Top Cascode Sources (M3 + M4): 9.70 μA+9.70 μA=19.40 μA

So Itot= 40 μA

CMFB

CD Buffers (M1 + M2): 2.43*2 μA=4.86 μA

Error Amp Tail: 4.73 μA

CMFB Subtotal: 9.59 μA

Total System Current (Itot): 40 μA+9.59 μA=49.6 μA

AREA

OTA

Input Pair (M0, M1): 2×20.88 μm×0.3 μm=12.53 μm2

Tail Source (M2): 1×17.68 μm×1.0 μm=17.68 μm2

Top Sources (M3, M4): 2×8.84 μm×1.0 μm=17.68 μm2

Top Cascodes (M5, M6): 2×10 μm×0.5 μm=10 μm2

Bottom Sinks (M9, M10): 2×5.52 μm×1.0 μm=11.04 μm2

Bottom Cascodes (M12, M13): 2×2.5 μm×0.5 μm=2.5 μm2

Total Core OTA Active Area: 71.43 μm2

CMFB

For the diff

Input Pair (M0, M1): 2×1.2 μm×1 μm=1.2 μm2

Tail Source (M2): 1×1.38 μm×1.0 μm=1.38 μm2

Top Sources (M3, M4): 2×2.21 μm×1.0 μm=4.42 μm2

For the sensing

CD: 2×6.2 μm×0.5 μm=6.2 μm2

Top CM : 2×2.21 μm×1.0 μm=4.42 μm2

Total CMFB Active Area: 17.62 μm2

TOTAL AREA = 89 μm2

FoM=UGFItot*AREA= 15.7 MHz89μ*49.6μ=3.6

That is very low to enhance it we will redesign the whole circuit so that the split ratio is not 1

We will set S = 3.5.