Information Technology Institute (ITI) Analog IC Design

# Lab 07

Five-Transistor OTA Design

Table of Contents

- Part 1: OTA Design (Hand Analysis)	2

- Part 2: Open-Loop OTA Simulation	6

  - DC operating point	6

  - Differential small-signal AC	7

  - Common-mode small-signal AC	9

  - CMRR	9

  - Avcm vs VICM (CMIR sweep)	10

  - Differential large-signal sweep	10

  - CM large-signal sweep (GBW vs VICM → actual CMIR)	11

- Part 3: Closed-Loop OTA Simulation (Unity-Gain Buffer)	12

  - Loop gain (STB analysis)	12

- Part 4: Effect of Mismatch on CMRR	13

# Part 1: OTA Design (Hand Analysis)

| Specs recap |  |
| --- | --- |
| Parameter | Value |
| Supply VDD | 1.8 V |
| Load CL | 5 pF |
| Open-loop DC gain | ≥ 34 dB |
| CMRR @ DC | ≥ 74 dB |
| Phase margin | ≥ 70◦ |
| Reference (seed) current | 10 µA |
| CMIR low | ≤ 1.2 V |
| CMIR high | ≥ 1.5 V |
| GBW | ≥ 5 MHz |
| Current consumption / Area | Minimize |

Q: NMOS or PMOS?

A:  NMOS, since the CMIR is closer to VDD we will choose nmos.

Bias current – simple approach

The dominant pole is at the output node, so GBW = gm1/(2πCL):

gm1 ≥ 2π · GBW · CL = 2π(5 MHz)(5 pF) = 157.1 µA/V

Assume gm/ID|1 = 15 as a first guess (moderate inversion):

gm1 	157.1

I	=	=	= 10.47 µA

so ID1 will be 10 µA

D1	gm/ID	15

157.1

gm/ID|1 =

= 15.7 S/A

10

Figure 1: W vs ID (left) and L vs ID (right) assume the tail has VDS,tail = 0.3 V, and plot Vicm,min = VGS1 + 0.3 vs ID:

Figure 2: Vicm,min = VGS1 + 0.3 V vs ID

At ID = 10 µA: Vicm,min = 1.127 V, which is below the 1.2 V.

Gain spec → sizing M3/M4 (load)

Av0

= gm1

(ro2

∥ ro4

gm1

) =		≥ 50.1 V/V (34 dB) 2λLID

gm1		157.1µ

λ  ≤	=

= 0.157 V −1, ro = 634kohm

L	2ID · 50.1	2(10µ)(50.1)

picking gm/ID|3,4 = 10 S/A for the load devices. Using CMIR-Max:

CMIRMAX = 1.5 = VDD + V gs1,2 − V dsat1,2 − V gs3,4 (Using V alue of V th2 from SA)

→ V gs3,4 = 1.023 ≈ 1V

CMRR spec → sizing M5 (tail)

CMRRspec − Av0,spec = 74 − 34 = 40 dB

CMRR = Av0 + 20 log10(2 gm3 ro5) ⇒ 2 gm3 ro5 = 100 ⇒ gm3ro5 = 50 With gm/ID|3 = 10 ⇒ gm3 = 100 µA/V :

ro5

50

=

100 µA/V

= 500 kΩ ⇒ λ5

1

=

ro5I5

1

=

(500k)(20µ)

= 0.1 V −1

assumed VDS,tail = 0.3 V, gm/ID = 10 for the tail.

Final sizing table

Table 1: Final transistor sizes and operating point (from ADT)

| Device | W (µm) | L (nm) | ID (µA) | gm (µA/V) | gm/ID (S/A) | V ∗ | (V) | Vov | (V) | Vdsat | (V) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M1, M2 | 10 | 450 | 10 | 157 | 15.76 | 0.128 |  | 0.031 |  | 0.104 |  |
| M3, M4 | 7.5 | 450 | 10 | 100.0 | 9.9 | 0.201 |  | 0.156 |  | 0.174 |  |
| M5 | 2 × 4.2 | 900 | 19.70 | 199.0 | 10 | 0.202 |  | 0.147 |  | 0.173 |  |
| M6 | 4.2 | 900 | 10.0 | 100.8 | 9.86 | 0.202 |  | 0.147 |  | 0.173 |  |

Figure 3: Tail / reference device (M5, M6) sizing

Figure 4: Input pair (M1/M2) sizing (left) and Load (M3/M4) sizing (right) The width of M1,2 is 8.5u but can be increased to 9.5u to achieve GBW=5MHz.

# Part 2: Open-Loop OTA Simulation

## DC operating point

Figure 5: Open-loop schematic with DC node voltages annotated (VICM = 1.35 V, middle of CMIR)

Q: Is the current and gm in the input pair exactly equal?

A: Yes, because the circuit is perfectly symmetric: both inputs have the same common-mode signal there’s no feedback to introduce any imbalance.

Figure 6: Ngspice op-point console output

Q: What is the DC voltage at VOUT? Why?

A: VOUT = 0.8653 V. it follows VF = V DD − VSG3 = 1.8 − 0.9347 = 0.8653 V, since the circuit is symmetric.

## Differential small-signal AC

Figure 7: |Av,diff | vs frequency (dB) (left) and ̸  Av,diff vs frequency (right) (gm1 = 155.26 µA/V , gds2 = 1.757 µA/V , gds4 = 0.658 µA/V ):

gm1		155.26

A	=	=	= 64.3 V/V = 36.16 dB

v,diff

gds2 + gds4

2.415

f	= gds2 + gds4 = 2.415 µA/V ≈ 76.9 kHz

p,out

2πCL

2π(5 pF )

gm1 	155.26 µA/V

GBW =	=

≈ 4.94 MHz

2πCL	2π(5 pF )

Since it’s a single dominant pole, PM should be close to 90◦.

Table 2: Differential AC – hand calc vs sim

| Quantity | Hand calc | Simulation |
| --- | --- | --- |
| Av,diff | 36.16 dB | 36.13 dB |
| BWdiff | 76.9 kHz | 76.1 kHz |
| GBW | 4.94 MHz | 4.877 MHz |
| PM | ≈ 90◦ | 90.51◦ |

Figure 8: Ngspice AC measurement summary

To obtain GBW larger we can increase the width of M1,2 to be 9.5u instead of 8.5u

Figure 9: Ngspice AC after increasing width

## Common-mode small-signal AC

Figure 10: |Av,cm| vs frequency (dB) (left) and ̸  Av,cm vs frequency (right) CM gain of a 5T OTA (matched devices) is approximately:

1

2 g

Av,cm ≈ −

m3

ro5

Using gm3 = 100.96 µA/V and ro5 = 1/gds5 = 1/1.0389 µA/V = 962.6 kΩ:

Av,cm

1

= −

2(100.96µ)(962.6k)

1

= −

194.4

= −5.14 × 10−3 = −45.8 dB

Table 3: Common-mode AC – hand calc vs sim

| Quantity | Hand calc | Simulation |
| --- | --- | --- |
| Av,cm | −45.8 dB | −47.97 dB |
| BWcm | 76.9 khz | 76.1 kHz |
| 2.4 CMRR |  |  |

Figure 11: CMRR vs frequency

CMRRhand = Av,diff (dB) − Av,cm(dB) = 36.16 − (−45.8) = 81.9 dB

Simulation gives CMRR = Av,diff db − Av,cm db = 36.13 − (−47.97) = 84.1 dB .

Table 4: CMRR – hand calc vs sim

| Quantity | Hand calc | Simulation |
| --- | --- | --- |
| CMRR | 82 dB | 84 dB |

## Avcm vs VICM (CMIR sweep)

Figure 12: Av,cm (dB, left) and (linear, right) vs VICM, swept across the CMIR

Q: Does Avcm increase or decrease vs VICM? Why?

A: It decreases As VICM goes up, VDS of the tail transistor M5 increases (it goes deeper into sat-uration), which increases ro5. Since Av,cm ∝ 1/ro5, a bigger ro5 means smaller CM gain, improving CMRR.

## Differential large-signal sweep

Figure 13: VOUT vs VID (left) and dVOUT /dVID vs VID (right)

Q: What is Vout at VID=0? Why? How does the derivative peak compare to Avd?

A: VOUT (VID = 0) = 0.8649 V as the open-loop DC bias point (0.8653 V).

The derivative dVOUT /dVID peaks at about 65 V/V at VID = 0, which is the gain Av,diff = 64.3 V/V , the derivative of the large-signal DC transfer curve at the origin is the small-signal gain.

## CM large-signal sweep (GBW vs VICM → actual CMIR)

Figure 14: GBW vs VICM, with 90%-of-max

The 90% threshold (≈4.41 MHz) is crossed at VICM=0.89 V on the low side and VICM=1.6 V on the high side, giving a simulated CMIR of about [0.89 V, 1.6 V].

# Part 3: Closed-Loop OTA Simulation (Unity-Gain Buffer)

Figure 15: Closed-loop DC OP annotated

Q: Is the current (and gm) in the input pair exactly equal ? Why?

A: No, ID(M 2) = 10.04 µA while ID(M 1) = 9.656 µA – a difference of about ∆ID ≈ 0.38 µA (∼3.9% mismatch ).

in the unity-gain buffer, VOUT follows VINP (=VICM= 1.35V) but only through a finite-gain loop, so it can’t track perfectly – VOUT settles at 1.343 V, not 1.350 V. That small difference becomes

a differential input Vid = VINP − VINN = 1.35 − 1.343 ≈ 7 mV . This Vid makes the differential pair

unbalanced like a mismatch.

## Loop gain (STB analysis)

Figure 16: Loop gain magnitude (dB) (left) and Loop gain phase (right)

Table 5: Loop gain vs open-loop dif f results

| Quantity | Open-loop (Part 2) | Loop gain (Part 3, STB) | Hand calc |
| --- | --- | --- | --- |
| DC gain | 36.13 dB | ≈36 dB | 36.16 dB |
| Crossover freq. | UGF = 4.894 MHz | GX = 4.898 MHz | GBW = 4.94 MHz |
| Phase margin | 90.51◦ | 90.51◦ (at GX) | ≈ 90◦ |

this is a unity-feedback buffer, so the feedback factor β = 1, and loop gain LG = β · Av,diff = Av,diff .

# Part 4: Effect of Mismatch on CMRR

Figure 17: ID1, ID2 vs VICM (overlaid)

For VICM below about 1.0 V, ID1 and ID2 are rising equally. Above ∼1.0 V they start to split apart: ID2 (red) keeps rising while ID1 (blue) is const(or we can say its deacreasing a little because the addition of them is const) 9.7 µA. this is because there is a mismatch in the input due to the feedback.

Figure 18: CM gain (linear) vs VIN

when VIcm is low the tail mosfet is not in sat so RSS is very small so the AVcm is large as VIcm increases RSS increases till The common-mode gain will reach its minimum best performance exactly at the point where VIN equals the voltage at the mirror node, because the input pair is perfectly balanced. also due to the feedback there is a systematic mismatch so the current differs.