INFORMATION TECHNOLOGY INSTITUTE

Analog IC Design Track

Analog Integrated Circuit Design

Lab 10

Noise Simulation

Table of Contents

PART 1: Feedback with Behavioral OTA	3

1.1	Closed-Loop Gain vs. Frequency	4

1.1.1	Simulation Results:	4

1.1.2	Hand Analysis:	5

1.2	Loop gain vs frequency (STB Analysis):	6

1.2.1	Simulation Results:	6

1.2.2	Hand Analysis:	7

1.3	Gain Desensitization:	9

1.3.1	Simulation Results:	9

1.3.2	Hand Analysis:	10

2	PART 2: Feedback with Real 5T OTA	11

2.1	Closed-Loop Gain vs. Frequency	11

2.1.1	Simulation Results:	11

2.2	Loop gain vs frequency (STB Analysis):	13

2.2.1	Simulation Results:	13

2.3	Gain Desensitization:	14

2.3.1	Simulation Results:	14

2.4	Transient analysis:	16

2.4.1	Simulation Results at FIN=1 kHz:	16

2.4.2	Simulation Results at FIN=BWCL (1.773 MHz):	18

- PART 1: LPF AC Noise Analysis

Output Noise vs. Frequency

Simulation Results:

Figure 1: output voltage noise spectral density vs frequency for  LPF (R=1 kΩ, C=1 pF)

As we see

Low-Frequency Voltage Noise Density: 4.07 nV/Hz

Bandwidth: 159.15 MHz

Hand Analysis:

Thermal Noise Density:

The input-referred thermal noise generated by the resistor is given by:

vn=4kTR

vn=4×1.38×10-23×300×1000≈4.07 nV/Hz

Bandwidth:

f3dB=12πRC

f3dB=12π100010-12≈159.15 MHz

Total RMS Noise (vn,rms):

vn,rms=kTC

vn,rms=1.38×10-23×30010-12≈64.34 μV

Figure 2: total RMS noise

Table 1: Comparison Table:

| Parameter | Hand Calc. | Simulation |
| --- | --- | --- |
| Noise Density | 4.07 nV/Hz | 4.071 nV/Hz |
| BW | 159.15 MHz | 159.15 MHz |
| Total RMS | 64.34 μV | 64.32 μV |

Parametric Sweep

Simulation Results:

Figure 3: Overlaid AC output noise spectral density plots for R=1kΩ,10kΩ,100kΩ, and 1MΩ

R=1 kΩ: ≈4.07 nV/Hz

R=10 kΩ: ≈12.87 nV/Hz

R=100 kΩ: ≈40.71 nV/Hz

R=1 MΩ: ≈128.25 nV/Hz

Figure 4: total RMS output noise for varying R

the total thermal noise of a simple RC circuit is independent of the resistance. Although a larger resistor generates a higher thermal noise spectral density (∝R), it proportionally restricts the noise bandwidth (∝1/R). During integration, the resistance terms cancel out, leaving the total noise voltage entirely constrained by the load capacitance (vn,rms=kT/C).

PART 2: LPF Transient Noise Analysis

Transient Noise Analysis

Simulation Results:

Figure 5: Transient noise output waveform

From the simulation plot:

- Maximum Value: 243.31 μV at 3.6 ns

- Minimum Value: -237.03 μV at 35.7 ns

RMS Noise Comparison

Figure 6: RMS calculation of the output noise

This value (63.53 μV) is close to the AC noise RMS value obtained in Part 1 (64.32 μV).

Effect of Time Step (TSTEP) on Noise:

The simulation was repeated with a larger time step, changing TSTEP from TAU/100 to TAU/10.

Figure 7: RMS calculation of the output noise with TSTEP=TAU/10.

Figure 8: Transient noise output waveform with (TSTEP=TAU/10).

Output Noise RMS Increased Slightly as Fmax increased so the area integrated is
bigger.

20 Iterations Analysis

Figure 9: RMS noise versus iteration number for 20 distinct transient noise simulation

Figure 1: Average RMS Noise

The averaged transient RMS noise (63.51 μV) is very closely to the AC simulation result from Part 1 (64.32 μV). although individual transient noise simulations are subject to high random variance.

PART 3: 5T OTA AC Noise Analysis

Thermal Noise vs. Frequency

Figure 11: Thermal noise

Thermal Voltage Noise Density: 15.62 nV/Hz

Closed-Loop Bandwidth: 4.896 MHz

Total Output Noise & Flicker Corner

Figure 12: Total output noise spectral density in log scale

Figure 13: corner frequency

Flicker Noise Corner Frequency (fc): 1.0 MHz

Figure 13: the total RMS noise (69.85 μV), bandwidth (4.896 MHz), and DC thermal noise floor (15.62 nV/Hz).

Figure 14: Integrated noise summary

Total Simulated Thermal Noise Power = 1.8887×10-9 V2

Simulated RMS Thermal Noise = 1.8887×10-9=43.46 μV

CL=5 pF, gm1,2=157 μA/V, and gm3,4=100 μA/V. assume γ≈2/3.

f3dB≈gm12πCL

f3dB≈157×10-62π5×10-12≈4.99 MHz

vn,th=16kT3gm11+gm3gm1

vn,th=161.38×10-233003157×10-61+100157≈15.16 nV/Hz

vn,rms,th=vn,th2×NBW=4kT3CL1+gm3gm1

vn,rms,th=41.38×10-2330035×10-121+100157≈42.5 μV

| Parameter | Hand Calculation | Simulation Result |
| --- | --- | --- |
| Bandwidth | 4.99 MHz | 4.896 MHz |
| Thermal Noise Density | 15.16 nV/Hz | 15.62 nV/Hz |
| Total RMS Noise | 42.5 μV (Thermal Only) | 43.46 μV |

PART 4: 5T OTA Transient Noise Analysis

Figure 15: 10 kHz, 10 mV amplitude input signal (yellow) and the resulting output signal (red) with a slight systematic DC offset

Figure 16: Full transient noisy output waveform

Figure 17: Zoomed-in view of the transient output

Figure 18: RMS vs iterations

From part 3 the thermal rms

vn,rms,th≈42.5 μV

Here the average is

vn,avg≈41.5 μV

Very close to each other as expected.